# TASK-2

TITLE: Design a 4-stage pipelined processor with basic instruction like ADD, SUB and LOAD

OBJECTIVE: 1. To design a basic 4-stage pipelined processor

2. To implement arithmetic and load instructions

3. To understand instruction-level parallelism

4. To simulate stage-by-stage execution using Verilog HDL

SOFTWARE USED:  vivado

PIPELINE ARCHITECTURE:

The processor contains 4 stages:

| Stage | Name | Function |
| --- | --- | --- |
| Stage-1 | Instruction fetch (IF) | Fetch instruction from memory |
| Stage-2 | Instruction decode (ID) | Decode instruction and read registers |
| Stage-3 | Execute (Ex) | Perform ALU operations |
| Stage-4 | Write Back (WB) | Store results into register |

Supported instructions:

| Instruction | Operation |
| --- | --- |
| ADD | Addition of two operands |
| SUB | subtraction of two operands |
| LOAD | Load data into register |

CODE:

VERILOG CODE FOR PIPELINE PROCESSOR:

`timescale 1ns / 1ps

///pineline processor design

module ppd(

input clk,

input reset

);

reg [7:0] regfile [0:7];

reg [15:0] instr_mem [0:15];

reg [7:0] data_mem [0:15];

reg [3:0] pc;

reg [15:0] if_id_instr;

reg [3:0] id_ex_opcode;

reg [3:0] id_ex_rd;

reg [7:0] id_ex_data1;

reg [7:0] id_ex_data2;

reg [3:0] ex_wb_rd;

reg [7:0] ex_wb_result;

integer i;

parameter ADD  = 4'b0001;

parameter SUB  = 4'b0010;

parameter LOAD = 4'b0011;

initial begin

regfile[0] = 8'd0;

regfile[1] = 8'd10;

regfile[2] = 8'd5;

regfile[3] = 8'd2;

regfile[4] = 8'd8;

regfile[5] = 8'd4;

regfile[6] = 8'd0;

regfile[7] = 8'd0;

data_mem[0] = 8'd25;

data_mem[1] = 8'd50;

instr_mem[0] = {ADD, 4'd0, 4'd1, 4'd2};

instr_mem[1] = {SUB, 4'd3, 4'd4, 4'd5};

instr_mem[2] = {LOAD, 4'd6, 4'd0, 4'd1};

end

always @(posedge clk or posedge reset)

begin

if(reset)

begin

pc <= 0;

if_id_instr <= 0;

id_ex_opcode <= 0;

id_ex_rd <= 0;

id_ex_data1 <= 0;

id_ex_data2 <= 0;

ex_wb_rd <= 0;

ex_wb_result <= 0;

end

else

begin

regfile[ex_wb_rd] <= ex_wb_result;

case(id_ex_opcode)

ADD:

begin

ex_wb_result <= id_ex_data1 + id_ex_data2;

ex_wb_rd <= id_ex_rd;

end

SUB:

begin

ex_wb_result <= id_ex_data1 - id_ex_data2;

ex_wb_rd <= id_ex_rd;

end

LOAD:

begin

ex_wb_result <= data_mem[id_ex_data2];

ex_wb_rd <= id_ex_rd;

end

default:

begin

ex_wb_result <= 0;

ex_wb_rd <= 0;

end

endcase

id_ex_opcode <= if_id_instr[15:12];

id_ex_rd     <= if_id_instr[11:8];

id_ex_data1  <= regfile[if_id_instr[7:4]];

id_ex_data2  <= if_id_instr[3:0];

if_id_instr <= instr_mem[pc];

pc <= pc + 1;

end

end

endmodule

TESTBENCH FOR THAT VERILOG CODE:

`timescale 1ns / 1ps

///testbench for the pipeline processor

module ppd_tb;

reg clk;

reg reset;

ppd uut(

.clk(clk),

.reset(reset)

);

always #5 clk = ~clk;

initial

begin

clk = 0;

reset = 1;

#10;

reset = 0;

#100;

$finish;

end

initial

begin

$monitor("TIME=%0t | PC=%d | R0=%d | R3=%d | R6=%d",

$time,

uut.pc,

uut.regfile[0],

uut.regfile[3],

uut.regfile[6]);

end

endmodule

PRACTICAL OUTPUT:

CONCLUSION:

The 4-stage pipelined processor was successfully designed and simulated using veilog HDL. The processor correctly execute ADD, SUB, an LOAD instructions through IF, ID, EX, and  WB stages